Schmitt Trigger Simulator

import streamlit as st

st.set_page_config(page_title="Schmitt Trigger Quiz", layout="centered")

st.title("Post-Lab Quiz: Schmitt Trigger using Op-Amp")

st.markdown("Test your understanding of Schmitt Trigger concepts.\n")

# MCQ Data with explanations

questions = [

{

"question": "What is the main purpose of a Schmitt Trigger circuit?",

"options": ["Amplify weak signals", "Convert AC to DC", "Remove noise from input signals", "Generate sine waves"],

"answer": 2,

"explanation": "Schmitt trigger removes noise by introducing hysteresis which converts noisy input into clean digital transitions."

},

{

"question": "What type of feedback is used in a Schmitt Trigger?",

"options": ["Negative feedback", "Positive feedback", "No feedback", "Current feedback"],

"answer": 1,

"explanation": "Positive feedback is used to create two different threshold voltages causing hysteresis."

},

{

"question": "Schmitt Trigger is basically a:",

"options": ["Comparator with negative feedback", "Amplifier with positive feedback", "Comparator with positive feedback", "Integrator"],

"answer": 2,

"explanation": "It is a comparator with positive feedback causing switching action with hysteresis."

},

{

"question": "What causes hysteresis in Schmitt Trigger?",

"options": ["Input bias current", "Slew rate", "Positive feedback", "Output saturation"],

"answer": 2,

"explanation": "Positive feedback shifts reference voltage, creating two different switching thresholds."

},

{

"question": "Hysteresis is the:",

"options": ["Output voltage", "Difference between threshold voltages", "Gain of op-amp", "Offset voltage"],

"answer": 1,

"explanation": "Hysteresis is the voltage difference between upper and lower threshold levels."

},

{

"question": "Which improves due to Schmitt Trigger?",

"options": ["Voltage gain", "Noise immunity", "Bandwidth", "Power dissipation"],

"answer": 1,

"explanation": "Noise margin is improved due to hysteresis effect."

},

{

"question": "Schmitt Trigger converts noisy signal into:",

"options": ["Sine wave", "Triangular wave", "Clean square wave", "DC signal"],

"answer": 2,

"explanation": "Any noisy waveform at input becomes a digital-like square wave at output."

},

{

"question": "In inverting Schmitt Trigger, input is given to:",

"options": ["Non-inverting terminal", "Inverting terminal", "Output terminal", "Ground"],

"answer": 1,

"explanation": "The input signal is applied to the inverting terminal for an inverting configuration."

},

{

"question": "Threshold voltage depends on:",

"options": ["Input frequency", "Supply voltage", "Feedback resistor ratio", "Load resistance"],

"answer": 2,

"explanation": "The ratio of feedback resistors determines the switching thresholds."

},

{

"question": "When Vin exceeds upper threshold, output:",

"options": ["Becomes zero", "Remains same", "Switches state", "Oscillates"],

"answer": 2,

"explanation": "Once Vin crosses threshold, op-amp output saturates to opposite rail."

}

]

# Initialize session state

if "score" not in st.session_state:

st.session_state.score = 0

if "current_q" not in st.session_state:

st.session_state.current_q = 0

if "answered" not in st.session_state:

st.session_state.answered = False

# ========== QUIZ LOGIC ==========

# If quiz finished

if st.session_state.current_q >= len(questions):

st.markdown("## 🎉 Quiz Completed!")

st.success(f"✅ Your Final Score: {st.session_state.score} / {len(questions)}")

if st.button("Restart Quiz"):

st.session_state.score = 0

st.session_state.current_q = 0

st.session_state.answered = False

st.experimental_rerun()

else:

# Display question

q = questions[st.session_state.current_q]

st.subheader(f"Question {st.session_state.current_q + 1} of {len(questions)}")

user_ans = st.radio(q["question"], q["options"], key=st.session_state.current_q)

# Submit Button

if st.button("Submit Answer"):

if not st.session_state.answered:

if q["options"].index(user_ans) == q["answer"]:

st.success("Correct! ✅")

st.session_state.score += 1

else:

st.error(f"Wrong ❌ | Correct Answer: {q['options'][q['answer']]}")

# Show Explanation

st.info(f"💡 Explanation: {q['explanation']}")

st.session_state.answered = True

# Next Button

if st.session_state.answered:

if st.button("Next Question"):

st.session_state.current_q += 1

st.session_state.answered = False

Active wave shaping circuit

import streamlit as st

st.set_page_config(page_title="Clipper & Clamper Quiz", layout="centered")

st.title("Post-Lab Quiz")

st.subheader("Clipper & Clamper using Op-Amp (Precision Circuits)")

# Question Bank with Explanations

questions = [

{"q": "Purpose of an op-amp in precision clipper is to:",

"opt": ["Increase bandwidth", "Remove diode drop effect", "Increase current", "Reduce gain"],

"ans": 1,

"exp": "Op-amp compensates for diode forward voltage drop, allowing accurate clipping even for very small signals."

},

{"q": "Clipping level in op-amp clipper is decided by:",

"opt": ["Load resistor", "Reference voltage", "Supply voltage", "Frequency"],

"ans": 1,

"exp": "External reference voltage sets the threshold at which clipping occurs."

},

{"q": "Double ended precision clipper:",

"opt": ["Clips one side", "Shifts signal", "Clips both sides", "Rectifies waveform"],

"ans": 2,

"exp": "A double ended clipper limits both positive and negative portions of the waveform."

},

{"q": "A biased clipper means:",

"opt": ["Clip at zero", "Clip at non-zero level", "Clip at peak", "No clipping"],

"ans": 1,

"exp": "Bias sets clipping level at a value other than zero volts."

},

{"q": "Major advantage of precision clipper is:",

"opt": ["Size", "High accuracy at low voltage", "Low cost", "No power supply"],

"ans": 1,

"exp": "It clips very small voltages accurately due to op-amp compensation."

},

{"q": "Main function of a clamper is to:",

"opt": ["Limit amplitude", "Shift DC level", "Amplify", "Invert signal"],

"ans": 1,

"exp": "Clamper adds or restores DC level without changing signal shape."

},

{"q": "Positive clamper shifts waveform:",

"opt": ["Upward", "Downward", "Clips positive", "Clips negative"],

"ans": 0,

"exp": "Positive clamper shifts entire waveform upward."

},

{"q": "Key element storing charge in clamper:",

"opt": ["Op-amp", "Resistor", "Capacitor", "Diode"],

"ans": 2,

"exp": "Capacitor stores charge to maintain DC shift."

},

{"q": "For proper clamping, RC should be:",

"opt": ["RC << T", "RC = T", "RC >> T", "RC < T"],

"ans": 2,

"exp": "RC must be much larger than signal period to prevent discharge."

},

{"q": "If capacitor fails in clamper:",

"opt": ["No effect", "No clamping happens", "Amplifies", "Clips input"],

"ans": 1,

"exp": "Without capacitor, DC level cannot be stored, so no shifting happens."

},

{"q": "DC restoration refers to:",

"opt": ["Amplitude control", "Baseline shifting", "Frequency increase", "Slope correction"],

"ans": 1,

"exp": "DC restoration re-establishes waveform baseline."

},

{"q": "Output amplitude in clamper is generally:",

"opt": ["Changed", "Reduced", "Constant", "Zero"],

"ans": 2,

"exp": "Clamping shifts waveform without altering peak-to-peak amplitude."

},

{"q": "Clipper vs Clamper:",

"opt": ["Both limit amplitude", "Clip limits, Clamper shifts", "Clamper limits, Clipper shifts", "Both amplify"],

"ans": 1,

"exp": "Clipper limits voltage, clamper shifts DC level."

},

{"q": "Precision wave shaping circuits mainly improve:",

"opt": ["Noise", "Accuracy", "Distortion", "Power"],

"ans": 1,

"exp": "They improve precision and waveform quality."

}

]

# Initialize Session

if "i" not in st.session_state:

st.session_state.i = 0

if "score" not in st.session_state:

st.session_state.score = 0

if "done" not in st.session_state:

st.session_state.done = False

# Display Quiz

if st.session_state.i < len(questions):

q = questions[st.session_state.i]

st.markdown(f"### Question {st.session_state.i + 1}/{len(questions)}")

selected = st.radio(q["q"], q["opt"], key=st.session_state.i)

if st.button("Submit"):

if not st.session_state.done:

if q["opt"].index(selected) == q["ans"]:

st.success("Correct ✅")

st.session_state.score += 1

else:

st.error(f"Wrong ❌ | Correct: {q['opt'][q['ans']]}")

# Show Explanation

st.info(f"💡 Explanation: {q['exp']}")

st.session_state.done = True

if st.session_state.done:

if st.button("Next"):

st.session_state.i += 1

st.session_state.done = False

else:

st.header("🎉 Quiz Completed")

st.success(f"Your Score: {st.session_state.score}/{len(questions)}")

if st.button("Restart Quiz"):

st.session_state.i = 0

st.session_state.score = 0

st.session_state.done = False

st.experimental_rerun()

Postlab MCQ of RC Phase Shift Oscillator Using Op-amp

import streamlit as st

st.set_page_config(page_title="RC Phase Shift Oscillator Quiz", layout="centered")

st.title("Post-Lab Quiz")

st.subheader("RC Phase Shift Oscillator using Op-Amp")

# Question Bank with Explanations

questions = [

{"q": "RC phase shift oscillator generates:",

"opt": ["Square wave", "Triangular wave", "Sine wave", "Sawtooth wave"],

"ans": 2,

"exp": "RC phase shift oscillator produces sinusoidal oscillations."

},

{"q": "Why is it called phase shift oscillator?",

"opt": ["It shifts amplitude", "It shifts phase using RC network", "It shifts voltage", "It shifts gain"],

"ans": 1,

"exp": "The RC network introduces a phase shift required for oscillation."

},

{"q": "Total phase shift required for oscillation:",

"opt": ["90°", "180°", "360°", "270°"],

"ans": 2,

"exp": "For sustained oscillations, total phase shift must be 0° or 360°."

},

{"q": "Number of RC sections normally used:",

"opt": ["1", "2", "3", "4"],

"ans": 2,

"exp": "Three RC sections are used, each contributing ~60° phase shift to total 180°."

},

{"q": "Phase shift provided by each RC section is about:",

"opt": ["30°", "45°", "60°", "90°"],

"ans": 2,

"exp": "Each RC network provides approximately 60° phase shift."

},

{"q": "Frequency of oscillation is:",

"opt": ["1/(2πRC)", "1/(2π√LC)", "1/(2πRC√6)", "1/(πRC)"],

"ans": 2,

"exp": "For 3 RC sections, oscillation frequency is f = 1 / (2πRC√6)."

},

{"q": "Minimum gain for oscillation is:",

"opt": ["10", "15", "29", "45"],

"ans": 2,

"exp": "Theoretical minimum gain is 29 due to RC attenuation."

},

{"q": "Gain is controlled by:",

"opt": ["RC network", "Feedback resistors", "Supply voltage", "Load resistance"],

"ans": 1,

"exp": "Gain is set by the resistor ratio in the op-amp feedback."

},

{"q": "If gain is reduced below minimum:",

"opt": ["Frequency increases", "No oscillations", "Output becomes square", "Noise reduces"],

"ans": 1,

"exp": "Oscillations stop if gain is insufficient."

},

{"q": "Increasing R or C decreases:",

"opt": ["Gain", "Voltage", "Frequency", "Power"],

"ans": 2,

"exp": "Increasing R or C reduces frequency since f ∝ 1/RC."

},

{"q": "Op-amp is operated in:",

"opt": ["Open loop", "Voltage follower", "Closed loop amplifier", "Saturation"],

"ans": 2,

"exp": "Closed loop ensures controlled gain."

},

{"q": "RC network provides:",

"opt": ["Gain", "Phase shift", "Power", "Bias"],

"ans": 1,

"exp": "RC network provides the required 180° phase shift."

},

{"q": "Barkhausen criterion:",

"opt": ["Gain = 1", "Phase = 180°", "Gain × feedback = 1 and phase = 360°", "Gain = 29"],

"ans": 2,

"exp": "For oscillations: loop gain = 1 and total phase shift = 360°."

},

{"q": "Removing one RC section will:",

"opt": ["Increase frequency", "Decrease output voltage", "Stop oscillation", "Increase gain"],

"ans": 2,

"exp": "Insufficient phase shift stops oscillations."

},

{"q": "RC oscillator is best suited for:",

"opt": ["RF range", "Audio frequency", "Microwave", "DC"],

"ans": 1,

"exp": "RC oscillators work best in audio frequency range."

}

]

# Initialize session variables

if "i" not in st.session_state:

st.session_state.i = 0

if "score" not in st.session_state:

st.session_state.score = 0

if "done" not in st.session_state:

st.session_state.done = False

# Quiz Navigation

if st.session_state.i < len(questions):

q = questions[st.session_state.i]

st.markdown(f"### Question {st.session_state.i+1}/{len(questions)}")

selected = st.radio(q["q"], q["opt"], key=st.session_state.i)

# Submit

if st.button("Submit"):

if not st.session_state.done:

if q["opt"].index(selected) == q["ans"]:

st.success("Correct ✅")

st.session_state.score += 1

else:

st.error(f"Wrong ❌ | Correct answer: {q['opt'][q['ans']]}")

# Explanation

st.info(f"💡 Explanation: {q['exp']}")

st.session_state.done = True

# Next

if st.session_state.done:

if st.button("Next"):

st.session_state.i += 1

st.session_state.done = False

# Result Page

else:

st.markdown("## 🎉 Quiz Completed")

st.success(f"Your Score: {st.session_state.score}/{len(questions)}")

if st.button("Restart Quiz"):

st.session_state.i = 0

st.session_state.score = 0

st.session_state.done = False

st.experimental_rerun()

Post lab MCQ of Wien Bridge Oscillator using op-amp

import streamlit as st

st.set_page_config(page_title="Wien Bridge Oscillator Quiz", layout="centered")

st.title("Post-Lab Quiz")

st.subheader("Wien Bridge Oscillator using Op-Amp")

# Question bank with explanations

questions = [

{"q": "Wien Bridge Oscillator generates:",

"opt": ["Square", "Triangular", "Sine", "Sawtooth"],

"ans": 2,

"exp": "It generates a sine wave output with low distortion."

},

{"q": "Main components used are:",

"opt": ["LC", "RC", "RLC", "Crystal"],

"ans": 1,

"exp": "Wien bridge oscillator is an RC oscillator, using resistors and capacitors for feedback."

},

{"q": "Phase shift at resonance is:",

"opt": ["90°", "180°", "0°", "360°"],

"ans": 2,

"exp": "At resonance, the phase shift across the RC network is zero degrees, satisfying the oscillation condition."

},

{"q": "Gain required for oscillation is:",

"opt": ["1", "2", "3", "29"],

"ans": 2,

"exp": "The loop gain must be at least 3 for sustained oscillations in the RC network."

},

{"q": "Frequency is f = 1 / (2πRC) when:",

"opt": ["R1=R2", "C1=C2", "R1=R2 and C1=C2", "Any values"],

"ans": 2,

"exp": "Frequency formula is valid when both resistors and capacitors in the feedback network are equal."

},

{"q": "The amplifier is operated as:",

"opt": ["Buffer", "Comparator", "Non-inverting amplifier", "Integrator"],

"ans": 2,

"exp": "Non-inverting amplifier provides the required gain and feedback for oscillation."

},

{"q": "If gain < 3, result is:",

"opt": ["No oscillation", "High distortion", "Frequency increase", "Clipping"],

"ans": 0,

"exp": "Insufficient gain cannot sustain oscillations."

},

{"q": "If gain > 3, result is:",

"opt": ["Stop oscillation", "Clipping/distortion", "Low output", "Frequency drop"],

"ans": 1,

"exp": "Excessive gain causes output distortion or clipping."

},

{"q": "Device used for gain stabilization:",

"opt": ["Zener diode", "Capacitor", "Bridge rectifier", "Lamp"],

"ans": 3,

"exp": "A lamp is used in the Wien bridge oscillator to stabilize the amplitude automatically."

},

{"q": "Lamp is used for:",

"opt": ["Phase shift", "Amplitude stabilization", "Voltage regulation", "Frequency increase"],

"ans": 1,

"exp": "The lamp adjusts resistance with temperature, stabilizing the amplitude."

},

{"q": "Best suited for:",

"opt": ["RF", "Microwave", "Audio frequency", "DC"],

"ans": 2,

"exp": "Wien bridge oscillator is ideal for audio frequency applications."

},

{"q": "Frequency depends mainly on:",

"opt": ["Supply voltage", "Op-amp", "RC components", "Load"],

"ans": 2,

"exp": "Oscillation frequency is determined by the RC network values."

},

{"q": "Low distortion is due to:",

"opt": ["RC only", "Lamp stabilization", "High gain", "Op-amp only"],

"ans": 1,

"exp": "Lamp-based automatic amplitude control ensures low distortion sine waves."

},

{"q": "At resonance feedback is:",

"opt": ["Minimum", "Maximum", "Zero", "Infinite"],

"ans": 1,

"exp": "Feedback is maximum at resonance, allowing sustained oscillations."

},

{"q": "Wien oscillator belongs to:",

"opt": ["LC", "RC", "Crystal", "Relaxation"],

"ans": 1,

"exp": "It is an RC oscillator using resistors and capacitors for frequency determination."

}

]

# Initialize session variables

if "i" not in st.session_state:

st.session_state.i = 0

if "score" not in st.session_state:

st.session_state.score = 0

if "done" not in st.session_state:

st.session_state.done = False

# Display Questions

if st.session_state.i < len(questions):

q = questions[st.session_state.i]

st.markdown(f"### Question {st.session_state.i + 1}/{len(questions)}")

selected = st.radio(q["q"], q["opt"], key=st.session_state.i)

if st.button("Submit"):

if not st.session_state.done:

if q["opt"].index(selected) == q["ans"]:

st.success("Correct ✅")

st.session_state.score += 1

else:

st.error(f"Wrong ❌ | Correct: {q['opt'][q['ans']]}")

# Show explanation

st.info(f"💡 Explanation: {q['exp']}")

st.session_state.done = True

if st.session_state.done:

if st.button("Next"):

st.session_state.i += 1

st.session_state.done = False

# Final results

else:

st.header("🎉 Quiz Completed")

st.success(f"Your Score: {st.session_state.score}/{len(questions)}")

if st.button("Restart Quiz"):

st.session_state.i = 0

st.session_state.score = 0

st.session_state.done = False

st.experimental_rerun()

Post Lab MCQ of Square Wave Generator using op-amp and also provide streamlit code

import streamlit as st

st.set_page_config(page_title="Square Wave Generator Quiz", layout="centered")

st.title("Post-Lab Quiz")

st.subheader("Square Wave Generator using Op-Amp")

# Question Bank with Explanations

questions = [

{"q": "Square wave generator is also known as:",

"opt": ["RC oscillator", "Relaxation oscillator", "LC oscillator", "Crystal oscillator"],

"ans": 1,

"exp": "It is called a relaxation oscillator because it relies on capacitor charging and discharging to generate oscillations."

},

{"q": "Main principle in square wave generator:",

"opt": ["Negative feedback", "Positive feedback with RC charging", "Phase shift", "LC resonance"],

"ans": 1,

"exp": "Positive feedback with capacitor charging/discharging creates the oscillation needed for square waves."

},

{"q": "Output waveform is:",

"opt": ["Sine wave", "Square wave", "Triangular wave", "Sawtooth wave"],

"ans": 1,

"exp": "The output is a square waveform, alternating between high and low levels."

},

{"q": "Comparator acts as:",

"opt": ["Integrator", "Amplifier", "Schmitt trigger", "Filter"],

"ans": 2,

"exp": "The comparator with hysteresis behaves as a Schmitt trigger to produce sharp transitions."

},

{"q": "Capacitor is used for:",

"opt": ["Voltage gain", "Charging and discharging", "Amplification", "Frequency boost"],

"ans": 1,

"exp": "The capacitor charges and discharges through the resistor to control timing and frequency."

},

{"q": "Output frequency depends on:",

"opt": ["R only", "C only", "Both R and C", "Supply voltage"],

"ans": 2,

"exp": "Frequency is determined by the RC time constant of the charging-discharging circuit."

},

{"q": "If capacitor value increases:",

"opt": ["Frequency increases", "Frequency decreases", "No change", "Stops"],

"ans": 1,

"exp": "Increasing capacitance increases the time constant, reducing oscillation frequency."

},

{"q": "Reducing resistance will:",

"opt": ["Decrease frequency", "Increase frequency", "Stop output", "No change"],

"ans": 1,

"exp": "Lower resistance reduces the RC time constant, increasing frequency."

},

{"q": "Duty cycle for ideal square wave:",

"opt": ["25%", "75%", "50%", "100%"],

"ans": 2,

"exp": "An ideal square wave has equal high and low times, so duty cycle is 50%."

},

{"q": "Output voltage swings between:",

"opt": ["±Vin", "±Threshold", "±Vsat", "0 to Vcc"],

"ans": 2,

"exp": "Output swings between the op-amp saturation voltages."

},

{"q": "Capacitor charging is:",

"opt": ["Linear", "Exponential", "Constant", "Zero"],

"ans": 1,

"exp": "Capacitor charges exponentially according to the RC time constant."

},

{"q": "Amplitude remains constant because:",

"opt": ["Negative feedback", "Threshold level", "Saturation voltage", "Temperature"],

"ans": 2,

"exp": "Op-amp output is limited by its saturation voltages, keeping amplitude constant."

},

{"q": "Feedback ratio affects:",

"opt": ["Duty cycle", "Frequency", "Amplitude", "Power"],

"ans": 0,

"exp": "Feedback affects how long the capacitor charges/discharges, influencing the duty cycle."

},

{"q": "Upper and lower thresholds are:",

"opt": ["Same", "Equal and opposite", "Zero", "DC"],

"ans": 1,

"exp": "Schmitt trigger thresholds are equal in magnitude but opposite in polarity for symmetric square waves."

},

{"q": "Square wave generator is used in:",

"opt": ["Timers", "Clock circuits", "Pulse generators", "All of these"],

"ans": 3,

"exp": "It is used in timers, clock generation, and pulse generation applications."

}

]

# Initialize Session State

if "i" not in st.session_state:

st.session_state.i = 0

if "score" not in st.session_state:

st.session_state.score = 0

if "done" not in st.session_state:

st.session_state.done = False

# Display Quiz

if st.session_state.i < len(questions):

q = questions[st.session_state.i]

st.markdown(f"### Question {st.session_state.i + 1}/{len(questions)}")

selected = st.radio(q["q"], q["opt"], key=st.session_state.i)

if st.button("Submit"):

if not st.session_state.done:

if q["opt"].index(selected) == q["ans"]:

st.success("Correct ✅")

st.session_state.score += 1

else:

st.error(f"Wrong ❌ | Correct Answer: {q['opt'][q['ans']]}")

# Show explanation

st.info(f"💡 Explanation: {q['exp']}")

st.session_state.done = True

if st.session_state.done:

if st.button("Next"):

st.session_state.i += 1

st.session_state.done = False

# Final results

else:

st.header("🎉 Quiz Complete!")

st.success(f"Your Score: {st.session_state.score}/{len(questions)}")

if st.button("Restart Quiz"):

st.session_state.i = 0

st.session_state.score = 0

st.session_state.done = False

st.experimental_rerun()

Post Lab MCQ of 1st order and 2nd order lowpass filter and also provide streamlit code

import streamlit as st

st.set_page_config(page_title="LPF & HPF Quiz", layout="centered")

st.title("Post-Lab Quiz")

st.subheader("1st Order LPF & HPF using Op-Amp")

# Question Bank with Explanations

questions = [

{"q": "Low-pass filter allows:", "opt": ["High", "Low", "All", "None"], "ans": 1,

"exp": "LPF allows low frequencies to pass while attenuating high frequencies."},

{"q": "At cut-off frequency, output becomes:", "opt": ["Zero", "Max", "0.707 times", "2 times"], "ans": 2,

"exp": "At cut-off frequency, output drops to 1/√2 (~0.707) of maximum."},

{"q": "Cut-off frequency formula of LPF:", "opt": ["1/RC", "1/2πRC", "πRC", "2πRC"], "ans": 1,

"exp": "The cut-off frequency for a first-order RC filter is fc = 1/(2πRC)."},

{"q": "Roll-off of LPF is:", "opt": ["-10", "-20", "-40", "-60"], "ans": 1,

"exp": "A first-order LPF has a slope of -20 dB/decade beyond cut-off."},

{"q": "Phase shift at fc (LPF):", "opt": ["0°", "-45°", "-90°", "-180°"], "ans": 1,

"exp": "At cut-off frequency, LPF introduces a phase shift of -45°."},

{"q": "Output at high frequency (LPF) is:", "opt": ["High", "Medium", "Zero", "Amplified"], "ans": 2,

"exp": "High frequencies are attenuated, so output approaches zero."},

{"q": "Op-amp provides:", "opt": ["Rectification", "Buffering & gain", "Modulation", "Switching"], "ans": 1,

"exp": "Active filters use op-amps for buffering and to provide gain without loading the RC network."},

{"q": "If R increases (LPF), fc:", "opt": ["Increases", "Decreases", "Same", "Doubles"], "ans": 1,

"exp": "Increasing R increases the RC time constant, lowering the cut-off frequency."},

{"q": "High-pass filter allows:", "opt": ["Low", "Medium", "High", "All"], "ans": 2,

"exp": "HPF allows high frequencies to pass while blocking low frequencies."},

{"q": "HPF output at low frequency:", "opt": ["Max", "Input", "Zero", "Infinite"], "ans": 2,

"exp": "Low frequencies are blocked, so output is nearly zero."},

{"q": "Cut-off frequency of HPF:", "opt": ["1/RC", "1/2πRC", "πRC", "2πRC"], "ans": 1,

"exp": "HPF cut-off frequency formula is the same: fc = 1/(2πRC)."},

{"q": "Roll-off rate of HPF:", "opt": ["+20 dB/dec", "-20 dB/dec", "0 dB/dec", "-40 dB/dec"], "ans": 0,

"exp": "HPF has a +20 dB/decade slope below the cut-off frequency."},

{"q": "Phase shift at fc (HPF):", "opt": ["0°", "+45°", "-45°", "90°"], "ans": 1,

"exp": "At cut-off, HPF introduces a phase shift of +45°."},

{"q": "High-pass filter blocks:", "opt": ["AC", "Noise", "DC", "Power"], "ans": 2,

"exp": "HPF blocks DC (0 Hz) while allowing AC signals to pass."},

{"q": "If C increases (HPF), fc:", "opt": ["Increases", "Decreases", "Same", "Zero"], "ans": 1,

"exp": "Increasing capacitance increases the RC time constant, lowering the cut-off frequency."},

{"q": "HPF provides:", "opt": ["Boost low f", "Boost high f", "Peak", "Notch"], "ans": 1,

"exp": "HPF passes high frequencies and attenuates low frequencies."},

{"q": "DC gain of HPF:", "opt": ["High", "Medium", "Low", "Zero"], "ans": 3,

"exp": "At DC (0 Hz), HPF output is zero."},

{"q": "Op-amp in HPF is used for:", "opt": ["Switching", "Impedance matching", "Modulation", "Detection"], "ans": 1,

"exp": "Op-amp provides buffering and impedance matching without affecting the RC network."},

{"q": "LPF & HPF have same formula because:", "opt": ["Same shape", "Same R", "Same RC constant", "Same output"], "ans": 2,

"exp": "Both filters use the same RC time constant formula to determine cut-off frequency."},

{"q": "Active filter advantage:", "opt": ["Bulky", "Needs L", "No gain", "Gain + isolation"], "ans": 3,

"exp": "Active filters provide gain and isolation without using inductors, making them compact and efficient."}

]

# Session State

if "i" not in st.session_state:

st.session_state.i = 0

if "score" not in st.session_state:

st.session_state.score = 0

if "done" not in st.session_state:

st.session_state.done = False

# Quiz Display

if st.session_state.i < len(questions):

q = questions[st.session_state.i]

st.markdown(f"### Question {st.session_state.i+1}/{len(questions)}")

selected = st.radio(q["q"], q["opt"], key=st.session_state.i)

if st.button("Submit"):

if not st.session_state.done:

if q["opt"].index(selected) == q["ans"]:

st.success("Correct ✅")

st.session_state.score += 1

else:

st.error(f"Wrong ❌ | Correct Answer: {q['opt'][q['ans']]}")

# Show explanation

st.info(f"💡 Explanation: {q['exp']}")

st.session_state.done = True

if st.session_state.done:

if st.button("Next"):

st.session_state.i += 1

st.session_state.done = False

# Final Result

else:

st.header("✅ Quiz Completed")

st.success(f"Your Score: {st.session_state.score}/{len(questions)}")

if st.button("Restart Quiz"):

st.session_state.i = 0

st.session_state.score = 0

st.session_state.done = False

st.experimental_rerun()